Solar Energy Activity (8/15/08)

Purpose

- This activity introduces Solar Energy and provide class activities for first year community college students or general audience.

Estimated Time

- Pre-class assessment (15 minutes)

- Solar Photovoltaic (30 minutes)

- Solar Photovoltaic Class Activity (15 minutes) with 45 minutes Homework

- Solar Thermal (30 minutes)

- Solar Thermal Class Activity (15 minutes)

- Post-class assessment (15 minutes)

Introduction

- Solar Energy Applications consists of two major technologies: Photovoltaic & Thermal

- This activity introduces the concepts of both technologies; instructor will guide students to form team of 2 to visit virtual tour (provided in power point) and work on practical applications.

Objectives

Students will be able to:

- List two ways to use solar energy to generate electricity

- List major components of a Photovoltaic Solar Power Plant

- List major components of a Thermal Solar Power Plant

- Describe operation of Inverter

- Describe operation of Transformer

- Describe two common Solar Thermal configurations to harness electricity for power utility companies

- Calculate Energy usage from electric bill

Attitude & Behavior

- Students must be active in learning, work as a team for class activity assignments and participate in team discussion and problem solving

Documentation

- Students shall document result in Power Point format as prescribed in class activity

- Report is judged by its accuracy and thoroughness

Equipment and Materials

- PC with Power point and projector

Resources

- http://en.wikipedia.org/wiki/Newton's_laws_of_motion

- Introduction to MEMS technology Power Point (with usage permission from Sandia National Lab)

- MEMS Sensors Power Point (with usage permission from Sandia National Lab)

- Experiment Data Samples (attached)

- Analog Device Accelerometer http://www.analog.com/en/cat/0,2878,764,00.html

Part I: Internet Research (Pre lab assignment):

Use available resources to define the following terms:

- Acceleration

- Force

- Mass

- Newton’s Second Law

- Newton’s Third Law

- Record results of your research in Student Response Sheet

Part II:  Analysis / Calculation (pre-lab assignment)

Given the following data on object A, calculate the force impact (unit in Newton)

| Mass (kg) | Acceleration (m/s/s) | Force (Newton) |
| --- | --- | --- |
| 0.5 | 5 |  |
| 0.75 | 3 |  |
| 1 | 15 |  |

Strategy:  Which laws of Physics do you apply to find the answer?

- Record your answers in Student Response Sheet

Part III: Set Up Lab experiments

Hardware Setup

- Connect Acceleration Sensor to PAScar

- Connect Sensor cable to GLX Xplorer

- Connect one end of USB cable to GLX Xplorer  and the other end to the USB port of a PC.

- Set up Mobile Track on a level table with one end rest on a 3 cm height support.

Software Setup (Data Studio)

- Run DataStudio, the following screen will pop up:

- Select Create Experiment

The following XplorerGLX File Manager will pop up:

- At XplorerGLX File Manager window, select RAM then Click Done

Set up Data Table

- At Displays menu, select Table

- The following menu will pop up:

- At Choose Data Source menu, select Acceleration, X, then click OK

The screen will display the following Table:

At the DataStudio menu, click Setup button:

- The following menu will pop up:

- Set Sample Rate at 10 Hz

- Check Zero sensor automatically on start

- At Experiment Setup window, Click on Sampling Options button, the following dialog box will pop up:

Set up Delayed Start Timer

At Sampling Options Menu, click on Delayed Start, select time = 1.5 seconds (or your preferred delayed start time) as indicated on the following menu, then click OK.

Set up Automatic Stop timer

At Sampling Options Menu, click on Automatic Stop, select time = 10 seconds (or the time duration for your data capture) as indicated on the following menu, then click OK.

Part IV: Running Experiments: Measure Force of Impact

1. Weigh the cart A (with Accelerometer installed) , paper box (3 lbs min.), cart B, Tennis Ball.

Record values in Data Table 1 on the Student Response Sheet.

2. Align the cart (with acceleration sensor) at the top end of Motion Track and hold the cart.

3. Take Run #1 Data: Measure Force of Impact on paper box without shock absorber

- At DataStudio screen,  Click the Start button.  When the Delayed Start Timer counts down to zero, release the cart. DataStudio will capture the acceleration data until the Automatic Stop Timer times out.

4. Take Run #2 Data: Measure Force of Impact on cart B without shock absorber

- At DataStudio screen,  Click the Start button.  When the Delayed Start Timer counts down to zero, release the cart. DataStudio will capture the acceleration data until the Automatic Stop Timer times out.

5. Take Run #3 Data: Measure Force of Impact on Tennis Ball without shock absorber

- At DataStudio screen,  Click the Start button.  When the Delayed Start Timer counts down to zero, release the cart. DataStudio will capture the acceleration data until the Automatic Stop Timer times out.

6. Extend the shock absorber at cart A.

7. Take Run #4 Data: Measure Force of Impact on paper box with shock absorber

Graphical Analysis

1.  Select Run #1 in the Graph Legend

2.  Click the Scale to Fit button to rescale the graph if needed

3. Record the value of the peak of acceleration and the associated time in Data Table on the Student Response Sheet

4. Repeat the process for Run #2, Run #3 and Run #4

Force calculations

Calculate the Force of Impact using Newton’s Second Law: F= ma

Where : 	m is mass (kg)

A is acceleration (m/s/s)

F is force (N)

Record results of calculations in Data Table on the Student response Sheet

Observations / Discussion

1. Explain why the Force of Impact is different between the 4 runs.

2. If you are using the accelerometer to detect collision between cart A and other objects. You want to activate an air bag to protect the passenger in the cart but you don’t want to have a false alarm when it hits a small object like tennis ball. What is the value of acceleration do you select to activate the air bag?