Short Case Studies

LOG 103

Case 1

Situation: The Hawkins Supply company is currently faced with an inventory rotation

problem. This difficulty stems from the fact that some supplies must be

used prior to a stated expiration date. Upon receipt, a new shipment of

these perishable items must be stacked beneath the boxes that are currently

in inventory. A substantial amount of time is consumed in restacking the

items according to their expiration dates.

Question: The company would like to reduce the double and sometimes triple

handling of items. How can this goal be achieved? Are there alternative

solutions which might also be effective?

Case 2

Situation: Field, Bell and Weiss, a consulting firm, has been engaged by the Fizzle

Beverage Company to determine possible methods for expanding their

warehouse facilities. The current warehouse has 16' ceilings with a possible

10' clear stacking height. At the 10' level the obstructions are steam pipes,

lighting fixtures and air ducts.

Fizzle Beverage currently receives all pallets by truck. Each pallet load is 6'

high (including the pallet). In order to take full advantage of all available

height the second level pallet in each stack must be broken down.

Question: How could Fizzle increase storage capacity?

Case 3

Situation: The Jones Company operates a centrally located storeroom in their

manufacturing complex. Every afternoon each craft foreman (Tin Shop,

Electric Shop, Iron Workers, etc.) writes a requisition for common use items

that will be required for the next day's work. These common use items

include nuts, bolts, screws, washers, flashlight batteries, and gloves. All

specialty items are ordered separately.

During the night shift, storeroom personnel fill the orders of items

requested by the craft foreman. Each morning, one or two workers from

each department go to the storeroom with a four-wheel platform truck to

pick up the filled order.

Question: Although studies have never been performed to determine the amount of

time craftsmen spend waiting for supplies, it is the thoughts of the

management that idle craft manpower is a problem resulting from this

procedure. How can time spent traveling to and from the described

storeroom be reduced, thus, eliminating or decreasing crafts' personnel

travel time?

Case 4

Situation: The White Manufacturing Company produces a spring-loaded replacement

spike for power rakes. Because of the small size of this item, they are

packaged in separate small containers that are in turn packed into a larger

carton (24 count) for shipping. The packing operation for this unit is on the

third floor of a multi-story building.

Upon completion of the packing operation the shipping cartons are placed

on semi-live skids and taken to the second floor using an elevator. The

same elevator is also used to move other materials to various floors in the

plant for processing. On the second floor packages are sorted according to

trucking line. After sorting, all packages are placed on a semi-live skid and

moved to the first floor via the same elevator. On the first floor, the

packages are stored awaiting shipment (pick up by the assigned truck line).

Question: Disregarding labor requirements, how can the movement of packages be

improved?

Case 5

Situation: The R-Way Distributing Company fills orders from one to one hundred

items ranging in size from 3" x 4" x 6" to 3' x 3' x 3'. All orders are put into

packing cartons and shipped by commercial carriers. The packaging

operation is performed by two workers on flat waist-high work branches.

Two workers are required because of the size of some of the cartons. To

secure the packages, rope, strapping, filament tape or gummed tape is

used. The wrapping operation is completely manual. After a package has

been packed, wrapped and sealed, it is hand-carried to classified shipping

bins about 400 feet from the wrapping tables.

Question: How can the packaging operation be improved? Suggest methods or

equipment for improving the handling of packages.

Case 6

Situation: The storage area of the P.D. George Company is presently filled to capacity

with 25,000 items. The plant has recently increased its manufacturing

capability by 100% and the finished goods inventory is expected to increase

by the same ratio. The present storeroom is 150' by 275' with a 30' ceiling

height.

Due to the features of the product line it is not advisable to stack the 15" x

25" x 12" cases more than 6 high without racks for support. The George

Company does not use racks in the inventory area.

Question: Suggest methods for increasing the storage area to accommodate the

contemplated increase in finished goods inventory.

Case 7

Situation: The Double Rite Bottling Company delivers soda to vending machines

throughout the northern section of Rhode Island. The Company takes pride

in the fact that all the beverages are sold in returnable bottles. The

president of Double Rite feels it is his civic duty to help decrease the

roadside trash that is often attributable to disposable beverage cans.

The returned empty bottles are stored on the roof of the truck when the

driver is making his deliveries.

Upon returning to the warehouse after deliveries are made, a helper passes

the empty cases from the roof of the truck to the driver standing on the

ground. The driver then places the cases onto a conveyor.

Question: How can the handling of empty cases from the roof of the truck be

accomplished in a more economical manner?

Disregard the handling of empty cases in the body of the truck.

Case 8

Situation: The Rosemarie Cosmetic company has a warehouse of 20 high pallet racks

serviced by counter-balanced fork trucks. The aisles between racks are 12

feet wide.

Storage space has become a premium with new product lines being added.

A warehouse expansion is currently being planned.

Question: How can the warehouse handling system be improved without expanding

the warehouse?