# Lesson 1: Overview of Remote Sensing

## INTRODUCTION

In this lesson you will learn about the various types of remotely sensed data and remote sensor systems used in remote sensing. The information will help you understand the uses of remote sensing such as monitoring cropland, forests for fires, wetlands, and floods. You will learn about the history of remote sensing as well as where the industry is today and where it is going in the future. This lesson provides information on the elements and strategies of visual interpretation and digital image processing. An overview of ArcGIS is included in this lesson to help you gain a better understanding of this software used for remote sensing.

## LESSON OBJECTIVES

By the end of this lesson, you will be able to:

- Define remote sensing.

- List different types of remotely sensed data and sensor systems.

- Describe different types of remote sensing systems and their applications.

- Describe the history and future trend applications of remote sensing.

## LEARNING SEQUENCE

| Required Reading | Read the following: Overview of Remote Sensing - Common Uses of Remote Sensing - Past, Present, Future of Remote Sensing - Overview of Visual Interpretation - Overview of Digital Image Processing - Overview of ArcGIS |
| --- | --- |
| Assignments | Complete the following assignments: - Application Paper - Quiz |

## INSTRUCTION

# Overview of Remote Sensing

## Remote Sensing

### What is Remote Sensing?

Remote sensing is basically observing the Earth and its phenomena using sensors, such as aerial and satellite systems that are not in contact with the Earth. An image analyst will use specialized software to study and analyze remotely sensed data. As part of these efforts image analysts will often create products such as maps, reports, and web services that are derived from these systems. Some examples include:

- Land cover classification map

- 3D elevation surface model that can be used in a viewshed analysis

- Image-based web map service that can be used in web and mobile mapping applications

Consider how remote sensing relates to other pieces of GIS and mapping. The image provided below illustrates how remote sensing relates to the other pieces of GIS and mapping. Remote sensing includes digital image processing, image acquisition, and photogrammetry, which is the science and processing of making highly refined corrections to aerial photos. Remote sensing connects with GIS when an analyst needs to make maps, conduct spatial analyses, and perform geoprocessing.

Remotely sensed data is often used as a background or as inputs to the geoprocessing functions within the GIS software. Remote sensing connects with GPS because the imagery can be used as a background in the GPS unit and provides a visual context when a person is using a GPS unit for data collection or navigation. Remote sensing connects with Internet mapping because again, the imagery can be used as a background and served up as a web map service. This can be used on web or mobile applications and provide visual context for these applications when they are used for navigation, routing, or for displaying other spatial data such as points of interest, traffic flow, areas of impact, etc. So as you can see remote sensing is an integral part of all aspects of GIS and location based information.

The lessons within this course will focus on remote sensing and include an introduction to a number of different kinds of imagery and various techniques to process the imagery to create and derive new spatial information.

## Common Uses of Remote Sensing

### Remote Sensing at Work

There are many different kinds of remotely sensed imagery. Remote sensing can be used for a wide variety of studies and fields of interest.

Some of the most common uses of remote sensing include:

- Land cover/land-use mapping

- Change detection which means looking at and analyzing change in the landscape over time

- Forest monitoring such as timber management, forest fires and bug infestations disease

- Urban sprawl and changes in the urban environment

- Natural resource management of many kinds

- Cultural resource analysis such as mapping historical archaeological sites where people lived and how they used the land from past cultures

- Fire monitoring such as forest fires or urban fires

- Feature material identification such as being able to categorize and identify specific objects in an image such as rooftops, buildings, trees, roads, and other types of objects

- Disaster management such as pre-and post-flood events, earthquakes, tsunamis, and other natural disasters as well as human impacts such as community and people displacement

- Climate change and looking at the effect of changes in the climate over the landscape

- Agricultural monitoring such as how different crops grow over growing season or how crops change over time

- Mining operations for both the actual mining operation as well as remediation of a mining site

- Flood monitoring of pre-and post-flood events such as are seen in hurricanes, localized flooding, levee breaks, and tsunamis

- Changes in snow and ice such as ice breakup, iceberg monitoring, and navigation of ships through Arctic or Antarctic waters

- Wetland management of being able to map wetlands as well as map changes in the wetlands

Other uses of remote sensing will be described and discussed throughout this course. You are encouraged to do your own investigation on the Internet and keep up with current events where remotely sensed imagery and data are used.

### Applied Uses of Remote Sensing

The following are examples of some of the uses of remotely sensed imagery and how this data can be incorporated into other applications.

#### Cropland Monitoring

Here is an example of cropland monitoring. This is a public website maintained by the USDA called CropScapes which is a web-based application that shows different types of crops that are grown throughout the United States. This data is updated annually and is derived from the MODIS and Landsat satellites. The general public and scientists can use this site to review, download data, and look at crop changes over time for different parts of the country and to do analysis on this information as needed.

CropScape – National Agriculture Statistics Service of the USDA use MODIS and Landsat

satellites to map and estimate crop type and monitor change in agriculture lands.

(http://nassgeodata.gmu.edu/CropScape/)

#### Monitoring: Forest and Fire

Remotely sensed imagery can be used for forest and fire monitoring. As can be seen in the imagery there are two large active fires burning. You can see the active fires burning as well as the area that has burned and the smoke plumes that are coming off the fires. In addition you will notice that there are clouds in the image. This image was taken from a Landsat 7 satellite in June 2002 in an area of Arizona. Since the Landsat satellite orbits in space the satellite not only picks up the fires and the smoke, but also picks up the clouds that are in scene.

US Forest Service – Rodeo and Chedeski Fires in Arizona from Landsat 7, June 21, 2002

(http://www.fs.fed.us/eng/rsac/)

### Monitoring: Surface Mining

Here is an example of how remote sensing can be used to monitor mining operations. The image on the left shows a person holding a remote controlled airplane that is considered an unmanned aerial vehicle or UAV and is used to fly around a mining site to monitor the mining operations. The image on the right happens to be a surface mine in West Virginia. In this case the USGS is flying the UAV mission to collect imagery and information for the mining operation.

USGS using UAVs for monitoring surface mining operations in West Virginia

(http://rmgsc.cr.usgs.gov/UAS/OSMMineSurveysWVGallery.shtml)

### Wetland Mapping

This image is an example of different kinds of wetlands that have been identified using satellite radar imagery. The map is a section of a large wetland in Alaska that has been categorized into different wetland types. This screenshot represents a small web-based mapping application created by Jet Propulsion Laboratory that shows the wetland mapped in Alaska.  See the URL within the slide to access the site of the application.

Alaska Wetlands derived from JERS RADAR satellite.

http://wetlands.jpl.nasa.gov/products/alaska_wetland.html

### Monitoring: Floods

The image below is an example of how remotely sensed imagery can be used for flood monitoring. The top image shows the Mississippi River in a pre-flood event and then the slide on the bottom shows the Mississippi River after a major flood that occurred in 1993. You can see how extensive the flood was in both Mississippi and Missouri rivers. This imagery and the maps derived from them were used to process insurance claims of affected property owners.

Mississippi River pre and post flood event.  Landsat 5 imagery was used to show the extent of flooding. Find out more about the Global Flood Atlas here: http://earthobservatory.nasa.gov/Features/HighWater/.

### Monitoring: Ice Conditions

This image illustrates how remotely sensed imagery can be used to monitor ice conditions. This image was taken by MODIS satellite in 2002 and in the center of the image there are two large icebergs near Antarctica. Remote sensing has also been used to monitor the Arctic and Antarctic sea ice freezing and thawing and the changing conditions of ice and snow as a result of climate change.

Ice monitoring showing two large ice bergs near Antarctica from the MODIS satellite, October 11, 2002

### Feature Based Mapping

Feature-based mapping or object-based image classification is a digital image processing technique that can be used to identify specific kinds of objects in high-resolution imagery. The image shown on the left represents a high-resolution satellite called Worldview-2. On the right, the picture shows the mapped land cover and land use types that were categorized using feature-based mapping methods. Visible are trees, shrubs, soil, buildings, roads, and other paved surfaces that have been identified in this image.

## Past, Present, Future of Remote Sensing

### History of Remote Sensing

Remote sensing has been around almost as long as photography itself. Chemical photography on a permanent medium has been around since the 1820’s. Shown here are some of the major milestones of remote sensing over the last 150 years.

1850’s: The first cameras used for remote sensing started in the late 1850’s with cameras attached to kites, balloons, and “pigeons”.

1903: With the advent of modern flight, remote sensing was used in World War I on reconnaissance flights to “see” where the various military posts were located. Since then aerial photography or aerial imaging is a standard practice for military purposes.

1940’s – Present: After the 1930s and 40s, aerial photography has been used to map and monitor natural resources such as agricultural lands, urban development, and changes in the natural environment.

1950’s -1960’s: During the development of the “Space Race” in the 1950s and 60s satellites were placed in orbit to test capabilities of communication and space flight.

1972: In 1972, the first satellite called the Earth Resource Technology Satellite (ERTS), and later renamed to Landsat-1, was put into orbit to monitor the Earth. Since then a number of government sponsored and private companies have placed Earth monitoring satellites into orbit. In February of 2013, the Landsat program marked its 40th anniversary with the launch of Landsat 8.

1980’s –Present: In the mid-1980s the US and other countries placed a variety of remote sensing satellites into space that include both multi-spectral and RADAR sensors.

1990’s – Present: In the 1990s to the present day, aerial and space-based remote sensors have improved both the spectral and spatial resolutions.  Some sensors such as Hyperspectral sensors can have over 200 unique spectral bands. Many airborne imaging systems can image with a ground resolution size of less than 3 inches, while commercial space-based imaging systems can capture image data in multiple wavelengths with a ground resolution of less than 1m. Also during this time, the development of LiDAR systems, sensors that capture high resolution elevation measurements using laser technology were also developed.

2010 – Present: In more recent and likely future times, the number and variety of satellite systems will continue to improve in both spectral and spatial resolution capabilities. In addition, the use of unmanned aerial vehicles (UAVs) and open source software have started to become popular and cost-effective for individuals and communities to capture, process, and use imagery from these systems as well as open new markets to offer new products and services that have in the past been cost prohibitive to develop and implement. Also, it is suspected that standards and regulations will be created and adopted by governments and agencies so that UAVs can be properly used for specific applications and balance the utility of such systems with privacy and “spying” issues.

### Timeline

Here is a timeline of the approximate history of the various technologies just mentioned. All of the various remote sensing systems have been in continuous operation since their inceptions are listed. The Balloon, kite, and “bird” options have had some sporadic history with some breaks here and there. Once motorized flight became common, the “low tech” options faded.

During the 40s to 60s, balloons were used in part to develop weather predicting technology as well as test photographic systems that were the precursors to “space” photography and the modern Earth imaging systems.

In recent times digital photography has become more common place with high quality resolutions.  The ability to mount and use low cost infrared digital imaging systems such as the kite and balloon mounted options have experienced a resurgence in interest and provide some opportunities for the “Do-It-Yourself” hobbyist to capture, process, and use “high tech” imaging systems. This was once limited to those with formal education and training in digital imaging systems and digital image processing techniques.

### Remote Sensing Technologies

Here are some examples of the remote sensing technologies just highlighted within the timeline.

1858 – Gaspard-Félix Tournachon (aka Nadar) French photographer who was the first person to take aerial photographs. This is an image of a photo taken by George Lawrence in 1906 of San Francisco about 6 weeks after an earthquake.  The camera was mounted to a kite.  Market Street begins at the “tower” in the foreground and disappears in the distance.  Golden Gate is in the upper right of the photo.

1906 – George Lawrence, San Francisco, kite mounted camera ~6 weeks after the 1906 San Francisco Earthquake. The tower of the Ferry building (foreground) marks the beginning of Market Street.  Golden Gate is towards the upper right where the sun is setting.

1908: Julius Neubronner, A pigeon mounted camera. This image is a representation of the “pigeon mounted” camera on the right. The left image shows one of the photos taken from this kind of camera.

1908 – Image taken by a pigeon mounted camera.

German trenches in WWI, 1916 in France. The vertical lines are the communication trenches. An aerial image taken from an airplane in France during World War 1.  The white linear features are those of the “trenches” that were dug where troops fought from.  The lines that run from the upper right to lower left represent the communication trenches where troops would send messages between military staff from one “line” to another.

### Images from Modern Satellite Systems

The examples of images provided were taken from modern satellite systems.

This is a Landsat scene of a portion of the Central Valley in California. Sacramento, CA is in the lower right quadrant of the image. The Landsat pixel size is 30m.

This is a subset of the commercial Quickbird. The Quickbird pixel size is 2ft. Notice the level of detail in the Quickbird image.

### High Resolution Digital Aerial Photography

The images provided illustrate an example of high resolution digital aerial photography and an example of airborne LiDar data.

This LiDAR image represents high resolution elevation data. This subset of downtown Sacramento shows the ancient course of the American River. The current American River runs across the top of the image. The “ancient” river system is now covered with large trees and a residential area and would not be recognizable in a typical aerial photo.

High Resolution Digital Aerial Photography                        Aerial LiDAR

### Future Remote Sensing Technology

Some examples of current unmanned aerial vehicles include “copters”, kites, fixed wing remote controlled air planes, and balloons. Many of these are low cost, (less than a couple of thousand dollars), and can have various digital camera systems mounted to them and typically include true color can color infrared imaging systems. Many are also programmed for flight and the imagery processed with open source software.

Examples of “low cost” UAVs for personal/community use.

## Overview of Visual Interpretation

### Interpreting Imagery

Part of remote sensing is to capture and process imagery. Other lessons that comprise this course will focus on the image processing aspects. One primary element of remote sensing is interpreting the imagery or the results of digital image processing routines. This is a key skill that image analysts develop and requires an understanding of the information collected by the sensor as well as the concepts and methods used to process the image data.

In addition to understanding the information collected by the sensor and analyzing the imagery, it is also helpful for the image analyst to have knowledge of the geographic area.  The analyst may not always work or live in the area that the imagery was taken, so obtaining a background on the geography or learning about the kinds of features one would expect is often helpful. Analysts may also need to look at a variety of information sources that may include images and derivative data sets from past projects in the same area, review sensor characteristics and image processing methods that could be used to process the image.

It is also common that the image quality may not always be adequate to visually interpret objects. Some issues that affect visual interpretation are haze, smoke, cloud cover, shadows from clouds, buildings, or other landscape features such as tree canopy or mountains.

Visual interpretation also often takes a lot of patience and imagination and be able to tie concepts of how the sensor collects spectral information to the pixels in the image that represent the geographic features.

### Performing Image Interpretation

Some common elements for performing image interpretation are as follows. These several of these elements are often used in any visual interpretation exercise and for any given type of imagery.

- Shape – this refers to the form, configuration or outline of an object. For example, a rectangular shape may represent a building

- Size – how large or small an object is relative to the image scale. Provided that the image has enough resolution to “identify” objects of interest, the object may appear as a small feature without much detail. In the case of high resolution imagery where there is a lot of detail in the image, the object of interest may appear large when the analyst zooms to the extent of the object

- Pattern – this refers to the spatial arrangement of the object. For example, an orchard or vineyard may appear in a regular arrangement of vegetation rows that represent the vines or the individual trees. A wetland may have an irregular pattern of open water and boggy vegetation areas

- Tone – tone refers to the relative brightness of an object for a given a color or black and white display. The analyst would expect building to appear bright in a true color image if the buildings are in an urban setting. Dark tones would be expected for the same urban areas when the image analyst is looking at an image that contains spectral information from color infrared imaging sensor.

- Texture – this refers to the spatial pattern of tone or color. Texture can play an important role in interpreting geographic objects in imagery. Some features will appear with a regular spacing of similarly colored or bright pixels; whereas others will tend to have a random pattern of pixels. For example, rows of corn in an image may have a regularly spaced set of bright pixels representing the corn and dark pixels representing the soil in between the corn rows. A wetland may have a random appearance of both dark and bright pixels.

- Shadows – shadows can assist the analyst to interpret features in the image. For example, shadows cast by palm trees may help to visually interpret palm trees.

- Site – the “site” of an object refers to its expected placement of this object based on the topography or geography that it is expected to be in. For example, certain kinds of trees grow at certain elevations or one would expect no trees above a certain elevation because some trees cannot grow above this elevation.

- Association – association refers to the placement of an object relative to other features. For example, wetlands might be expected to be found near rivers, streams, or creeks.

- Resolution – the image resolution limits the kinds of features an image analyst can interpret. For example, if an image has a 30m pixel resolution, it is likely not possible to identify individual buildings or houses. Whereas, in a high resolution image with 6in pixels, the details of houses and buildings can be identified (such as windows, gables, and vents).

### Interpretation Strategies

Some of the general interpretation strategies that image analysts tend to use in any visual interpretation exercise are explained here.

#### Direct Observation

Direct Observation refers to actually “looking at” the imagery and interpreting the object and being able to identify it. For example, the feature I look at on an urban image is a high rise office building.

#### Inference of an Object

This concept involves an image analyst who can infer what the object is in the image by looking at surrounding geography and other nearby objects. For example, a specific kind of forest plant community can be inferred by its presence on a specific facing slope (such as South facing) of a hill.

#### Knowledge of the Area

It is often helpful to have knowledge of the geography and surrounding area when working on a remote sensing project. If the analyst does not live or work in the local area, then he or she may need to visit the area, talk to others who live or work there, and obtain and study other reference documents and information on the geography.

#### Reference Material

This includes various kinds of digital or paper information that can include maps, image classifications of land cover, stream and river networks, wetland maps, urban infrastructure, and acquiring digital geographic reference information such as streets, lake boundaries, forest stand information, trails, etc.

## Digital Image Processing

### Overview of the Digital Imaging Process

Another major part of remote sensing is the use of digital image processing techniques to create and derive new information which can be other images, tables, or maps. Digital image processing is at the heart of remote sensing, since specialized software is required to obtain, analyze, and interpret information. Many different digital image processing software packages are available that can be used to process remotely sensed imagery.  In this course, ArcGIS will be used. Other commercial software that is available are ERDAS Imagine, ENVI, and PCI Geomatics. Some popular open source software packages are Opticks and MultiSpec.

The digital image processing techniques involve running “computer algorithms” that operate over all or parts of the image and depend on what parameters are required and how the software vendor has chosen to implement these algorithms.

An Algorithm is essentially one or more math functions that are used to analyze the pixel values within in an image. The input data for these math functions can be individual pixels, specific image bands, subsets of pixels, or the entire image. Unit 3 will focus on image components and the relationship between sensor wavelengths, the image, and some of the physical properties that can be derived from the image. Other units will focused on other involved processes such as land cover classification.

### Digital Imaging Processing Routines

Some of the common digital image processing routines can be categorized in the following manner.

#### Data Composition

Data Composition refers to creating a multi-band image composite from individual sensor bands, creating image mosaics from multiple images, or subsets of images which might be a portion of an image that represents a project area for a mapping or image classification project.  Data composition tasks are often considered a pre-processing step since an image (or images) needs to be compiled and made available so that other image processing steps can take place.

#### Image Rectification

Image Rectification this involves the process of locating an image without any spatial reference to geographic data that does contain a spatial reference. Image rectification is also considered a pre-processing step, since most remotely sensed imagery of the Earth needs to be referenced to an actual geographic location so that it can be used with other georeferenced data.

#### Filters

Many image processing packages provide s series of image filters that can be used to smooth or identify linear objects such as roads or crop boundaries.

#### Image Classification

A major task using remotely sensed imagery is being able to categorize land cover or land use.  A number of image classification techniques exist for semi-automatically or automatically being able to categorize land cover or land use. These processes often involve a number of sequential steps and analyses to complete.

#### Derive New Data

In many cases, new data sets are often derived from the raw remotely sensed imagery. Some of these data may exist as temporary data for multi-step processes or they can be used by themselves or as inputs to modeling processes. Examples of these data sets are slope, aspect, texture, band ratios, and vegetation indices.

#### Image Fusion

Image Fusion is the process of merging imagery from different sensor types or different resolutions. Examples include high resolution color composites and merging RADAR data with multi-spectral data.

#### Data Conversion

A common activity that is used with remotely sensed imagery is changing the image format from type to another. This can include different file types (such as converting from an ERDAS Imagine file format to a TIF format).  It can also include converting from an uncompressed format to a compressed format (or vice versa). Performing image compression often requires the use of additional commercial software extensions to image processing or GIS packages.  Examples include the MrSID format from LizarTech and the ECW format created by Earth Resource Mapping and is now part of Integraph.

#### Special Algorithms

These include one or more related computer routines that are specific to unique types of remotely sensed data such as RADAR, Hyperspectral, or LiDAR. Often these routines are “add-on” packages to existing image processing or GIS software.

### Digital Image Processing

Here are some examples of how the digital image processing routines can be implemented.

#### Tools with Parameters

Most often, the analyst will chose a routine that will pop-up a dialog box. The analyst will then need to fill in parameters such as inputs, output, and other specific parameters that are required or are optional for the routine to function properly. When the analyst clicks OK, the image will be processed with these parameters. The new image file can then be analyzed and visually interpreted or have other subsequent processes run on it.

#### Spatial Models

Another method to process image data is to use spatial models. In the case of ArcGIS this is accomplished through ModelBuilder. In other software packages this is achieved through a similar means of graphical objects being placed and connected in a “canvas” area to build the spatial model. The analyst can test and then run the model through a “Run” operation or tool.

#### Scripts

A third method of processing imagery is through writing computer code. In some cases, the tools are not available through the “out-of-the-box” software or add-on extensions. In these situations and provided that the analyst has the required programming skills, the analyst can write computer algorithms and image processing routines to process the image. If the analyst does not have the programming background to accomplish this, they can often partner with another person or organization that can perform the unique programming tasks.

In some cases, the software vendor will have its own unique software tools to write and process the computer code. In many of the current commercial software packages, Python, C++, or similar common programming language is used to write specific image processing scripts.

## Overview of ArcGIS

### Introducing ArcGIS

ArcGIS will briefly be introduced as the software that will be used in this Remote Sensing course. The current version of ArcGIS 10.1 will be used. Older versions of ArcGIS can be used, but not all of the functionality for performing the image processing tasks in this course may be available.

As a student you should already be familiar with the basics of ArcGIS that were acquired in other foundational geospatial technology courses. Students are encouraged to review these courses and materials to gain a functional background with using ArcGIS and its components.

### Major Components of ArcGIS

ArcGIS software has three major components.

#### ArcMap

ArcMap provides the ability to perform map design, data processing, and data analysis work.  This module is where analysts will spend most of their time interacting and interpreting data.

#### ArcToolbox

The ArcToolbox provides the “geoprocessing” ability to perform image and vector based geographic analysis. Analysts run specific tools from the toolbox to carry out the image processing. ArcToolbox can process a multitude of raster, vector, and tabular data.

#### ArcCatalog

ArcCatalog is the explorer for spatial data. ArcCatalog is set up similar to the Windows Explorer, but is tailored for creating and managing spatial data sets. In many cases analysts will create new data sets, copy and paste data from one file location to another, and view properties of data sets through ArcCatalog.

### ArcMap and ArcToolbox

Data can be loaded into the viewing window so the analyst can see the specific spatial data. Located on the right side of the viewing window is the table of contents that the analyst can use to turn on or off data as well as change colors, labels, and image band combinations. See the tab at the lower left named Table of Contents.

In addition, some tools may appear on top of the viewing window where the analyst can interact with the tool to change parameters or options for the image or data set.

Also shown in this slide is the ArcToolbox. The list of tools is shown on the left side of the viewing window. The ArcToolbox can be “docked” in any location within ArcMap and can also appear as a vertical tab that can often be found on the right side of ArcMap. Toolboxes can open ToolSets and ultimately Tools that the analyst will add the parameters to perform image processing.

### ArcCatalog

In this image of an ArcCatalog notice how it is organized by a number of folders that contain different kinds of spatial data sets. ArcCatalog requires “Data Connections” to be made so that the contents within the folders can be accessed. As can be seen in this image, the analyst can click on a folder and see the contents within it. The right side of ArcCatalog provides some basic details of the data types and formats. Analyst can also “right-click” the mouse and review other properties of the data sets.

In addition, the analyst can click on the Preview Tab to see an overview of the spatial data as well as preview the tabular data behind the geographic data. The analyst can also click on the Details Tab that will show the “metadata” about the data set. The metadata can provide useful information such as a short abstract summary of the data, who created the data, how it was compiled, the spatial reference of the data, when it was updated, and restrictions of use.

### ArcGIS Overview

In this course a number of components of ArcGIS will be used to perform various kinds of image processing functions. The Image Analyst Window will be used to perform some of the preprocessing functions such as image subsets, mosaics, image composition from individual sensor bands, etc. The way to access the Image Analysis Window is through the Window menu option on ArcGIS, then click Image Analysis. The window shown on the right side of the viewing window will appear.

The toolbars can be activated by right-clicking in the toolbar area towards the top of ArcGIS.  In order for the 3D and Spatial Analyst toolbars and toolboxes to function, the extension must be turned on and can be done by clicking on Customize—Extensions. Remember, the extensions must be purchased and installed in order for ArcGIS to allow this functionality. The extensions must also be activated separately in both ArcMap and ArcCatalog. Other units in this course will provide more details on using some of the tools within each of these toolbars or toolboxes.

### ArcGIS Extensions

In addition, a couple of ArcGIS Extensions will be used:

*Spatial Analyst– the spatial analyst extension will be used to perform some of the image processing functions such as image classification, map algebra, and some raster modeling processes such as creating digital elevation models and slope and aspect layers

*3D Analyst – the 3D Analyst extension will be used to perform operations on elevation data such as LiDAR.

NOTE:  The asterisk indicates that these extensions will require an additional purchase to the ArcGIS software.

Toolbars and ArcToolboxes

A number of Toolbars and ArcToolboxes will also be used. Some of them are shown here.

- Image Classification Toolbar provides the ability to generate semi-automated and automated land cover or land use data sets.

- 3D Analyst Toolbar and tools provides some tools for processing LiDAR and other terrain data.

- LAS Dataset is a toolbar and toolset that allows for some additional LiDAR data processing

- Georeferencing Toolbar provides the ability to perform image rectification

- Spatial Analyst is a toolbar and toolbox that allows for a variety of image processing and raster modeling tasks.

## Summary

In this lesson you learned about the various types of remotely sensed data and remote sensor systems used in remote sensing. The information helped you understand the uses of remote sensing such as monitoring cropland, forests for fires, wetlands, and floods. You learned about the history of remote sensing as well as where the industry is today and where it is going in the future. This lesson provided information on the elements and strategies of visual interpretation and digital image processing. An overview of ArcGIS was included in this lesson to help you gain a better understanding this software used for remote sensing.

## Assignments

- Application Paper

- Quiz