ACADs (08-006)  Covered

| 3.3.9.8.1 | 3.3.9.8.2 | 3.3.9.8.3 | 3.3.9.8.4 | 3.3.9.8.5 | 3.3.9.17 | 3.3.12.14 |
| --- | --- | --- | --- | --- | --- | --- |
| 3.3.13.6 | 3.3.13.19 | 3.3.13.21 | 3.3.13.23 |  |  |  |

Keywords

Decontamination, special precautions, wet mop heads, liquids containing detergents, vacuum cleaner dust, mixed waste.

Description

This document provides references to readings and other resources to support a lesson on how to determine dose to skin, as well as provides scenarios and questions to engage students.

Supporting Material

Spills and Contamination Events

# Module 3: Spills and Contamination Events

Table of Contents:

Resources Key	2

Module Readings and Homework	2

Primary Scenario “Radioactive Water Spill Due To Damaged Hose Inside a Radwaste Facility”	2

Transfer Scenario “Demineralizer Filter Backwash Water Tank Overflows”	2

Primary Scenario “Liquid Spill During Sludge Transfer from Steam Generator Blow-Down Tank”	3

Transfer Scenario “Floor Drain Pipe Restriction Results in Overflow of Water”	3

Transfer Scenario Original Event Descriptions	4

Transfer Scenario “Demineralizer Filter Backwash Water Tank Overflows”	4

Transfer Scenario “Floor Drain Pipe Restriction Results in Overflow of Water”	5

Module Assessment Items	6

Primary Scenario “Radioactive Water Spill Due To Damaged Hose Inside a Radwaste Facility”	6

ACAD References	8

# Resources Key

| This refers to: | This reference: |
| --- | --- |
| ACAD | National Academy for Nuclear Training, Uniform Curriculum Guide for Nuclear Power Plant Technician, Maintenance, and Nonlicensed Operations Personnel Associate Degree Programs, ACAD 08-006. |
| DOE-SG | Office of Environmental, Safety and Health: Radiological Control Technician Training Site Academic Training Study Guide Phase I, Project Number TRNG-0003 Available at: http://nsedu.rnet.missouri.edu/docshare/. File is located under the Docs/General Curriculum/DOE materials folder. |
| G. | Gollnick, D. (2006). Basic Radiation Protection Technology, 5th Ed. Pacific Radiation Corporation, Altadena, CA. |

# Module Readings and Homework

Primary Scenario “Radioactive Water Spill Due To Damaged Hose Inside a Radwaste Facility”

| Core Concept: Techniques and precautions for minimizing exposure from spills and contamination |  |  |
| --- | --- | --- |
|  | Homework |  |
| Readings | Calculation Items | Non-calculation Items |
| DOE-SG-Mod 2.13-9 to 2.13-10 | N\A | N\A |
| Core Concept: Techniques and precautions for controlling the spread of contamination |  |  |
|  | Homework |  |
| Readings | Calculation Items | Non-calculation Items |
| DOE-SG-Mod 2.05-5 to 2.05-8 | N\A | DOE-SG-Mod 2.05.03 to 2.05.05 |

Transfer Scenario “Demineralizer Filter Backwash Water Tank Overflows”

- Refer to readings and homework for primary scenario above.

- The complete text of the event for this scenario is in the section “Transfer Scenario Original Event Descriptions” below.

Primary Scenario “Liquid Spill During Sludge Transfer from Steam Generator Blow-Down Tank”

| Core Concept: Techniques and precautions for controlling liquid spills |  |  |
| --- | --- | --- |
|  | Homework |  |
| Readings | Calculation Items | Non-calculation Items |
| N\A | N\A | N\A |
| Core Concept: Equipment used for containment of contamination |  |  |
|  | Homework |  |
| Readings | Calculation Items | Non-calculation Items |
| G., Chap. 14, 627-629 | N\A | N\A |

Transfer Scenario “Floor Drain Pipe Restriction Results in Overflow of Water”

- Refer to readings and homework for primary scenario above.

- The complete text of the event for this scenario is in the section “Transfer Scenario Original Event Descriptions” below.

# Transfer Scenario Original Event Descriptions

Use the complete text of these events to assess students’ ability to analyze the event, and apply appropriate safety and response procedures.

Transfer Scenario “Demineralizer Filter Backwash Water Tank Overflows”

Description:  One of the reactor water clean-up (RWCU) demineralizer filters was being backwashed. When the backwash was initiated the level in the backwash water tank was approximately 40% as indicated in the corresponding RWCU panel.

Backwash water caused the backwash water tank to overflow. Part of the overflowing water ran to the containment sumps and part of it went to the containment HVAC exhaust duct, where the process radiation monitors are located.  High radiation alarm was activated and all four process radiation monitors tripped, thus causing automatic startup of both trains of the standby gas treatment system (SGTS) and containment building HVAC isolation.

Radiation protection personnel were sent to the area where the containment HVAC radiation monitors were located and they found a high dose rate in the HVAC exhaust duct which indicated the presence of contaminated water inside. Mechanical personnel removed the inspection cover of the HVAC duct this allowing personnel to reach the contaminated water which was immediately suctioned and pumped to the containment sump.

The maximum radioactivity released outside containment as read by the effluent gas monitors located at the ventilation exhaust stack was 2.5E-6 microcurie/cc for a brief interval then returning to the normal values rapidly.

Causes:  The cause of the backwash water tank overflowing was the level indication on the RWCU had been erroneous, as the backwash water tank level indication in the Radwaste Treatment Building Control Room was 80% which was the correct value as it could be verified later.

Corrective Actions:  The RWCU panel level indicator was not properly adjusted so it was corrected.  A label has been put onto the RWCU panel warning not to start the demineralizer filters backwash without previously verifying that the backwash water tanks are empty as indicated by both indicators.  A design change has been implemented so that the interlock setpoint for the backwash water tank has been reduced to 40%. Above that level filter backwash is now automatically blocked.

Transfer Scenario “Floor Drain Pipe Restriction Results in Overflow of Water”

Description:  Heat removal system boundary valves were closed and draining was commenced in preparation for performance of leak testing the shutdown cooling system isolation valves. Low point drains directed to the floor drain sump were opened first, followed by drains in the mid level of the NW quad which are also directed to the floor drain sump. At this point a vent valve in the NW quad mid level was opened and directed to a floor drain through a hose already installed and tagged by RP. Air was observed to be flowing into the hose as expected when draining. The piping drain-down was monitored at both drain points outside of the drywell, and was determined to be well within the capacity of the selected drain paths.

The team inside the drywell was then notified to re-position valves and hang the remaining tags on vents and drains in the drywell. This apparently increased the rate of drainage in the lower elevations of piping outside the drywell, resulting in water draining from the vent line in the mid-level of the NW quad.  Initially, air had been flowing into the system through this path, while the water drained out of the system, as expected. However, the floor drain this vent hose was directed to was apparently partially plugged. As a result, some of the water drainage volume overflowed the floor drain and began spilling onto the floor. Some of the overflowing water went through another floor drain to the floor drain tank. The rest of the water flowing onto the floor, flowed over to a floor grating between the mid-level and the bottom level of the quadrant, and began running through this grating to the lower level where the heat removal pumps are located. 

Cause:  Based on the fact that no drainage problems were encountered during draining from the lower elevation, it can be surmised that the floor drain in the mid level of the NW quad has a restriction that caused drainage flow to backup and spill over onto the deck grating.

Corrective Action:  With RP coverage the vent hose was moved to a nearby equipment drain and the vent valve opened.  Air was observed to be sucking into the hose as expected for a drain operation. Other drain paths were walked down to ensure that there were no similar problems in other areas.  None were noted. Subsequently, the pump motors were evaluated and determined to not have suffered any damage.

# Module Assessment Items

Note: If instructors wish to increase the difficulty of any item, then we suggest you use it as the basis for an in-class discussion, and / or require students to write an explanation for why a particular choice is correct.

Primary Scenario “Radioactive Water Spill Due To Damaged Hose Inside a Radwaste Facility”

Bob is a Radiation Protection Technician (RPT) at the Blue Lake Nuclear Plant. Bob’s first task for the day is to complete a radiation survey of the “B” RHR pump room. After reviewing the last survey, Bob obtained a clipboard, a survey map, and a portable dose rate meter equipped with an ionization detector. He checked its condition, calibration, and response check. Everything looked OK. As he was walking through the main hallway of the Auxiliary Building, he noticed drops of water on the floor. Being careful not to step into the water, Bob immediately checked it with the portable dose rate meter and observed no appreciable dose rate increase (<0.5 mrem/hr) so he concluded it was not contaminated.

As he examined the water closer, he saw that it continued down the hallway and around the corner. Once around the corner, Bob saw water dripping from a valve and puddling in the hallway. He also checked the puddle with his dose rate meter and noticed no appreciable dose rate increase (<0.5 mrem/hr). He wasn’t sure what system the valve was part of or what position it should be in. The water appeared to be coming from the packing gland around the valve stem at about 10 drops/min. Since no one appeared to be in the area at the time, Bob attempted to stop the leak by changing the position of the valve, but he was not sure whether the valve was open or closed. As he tried closing the valve tightly against its seat the leakage increased slightly to about 1 drop/sec. He then tried opening the valve all the way to place the valve disc against the back seat to stop the leak. The increased leakage continued. Jim Black, an Equipment Operator walked around the corner, not noticing the water on the floor and, as he saw Bob operating the valve, he stepped in the puddle of water. He asked Bob, “What’s going on? Why are you operating that RHR valve?” Bob responded, “It seems to be leaking around the stem and I’m trying to stop it.” Jim said, “That water is from the RHR system and has contamination in it.”

- (Inference) What should Bob do next?

  - Ask Jim to get some absorbent rags to soak up the water while he keeps trying to stop the leak

  - Ask Jim to go get a wrench to tighten down the valve packing gland

  - Ask Jim to stop and don’t move while he contacts the Control Room and gets some help (correct)

  - Ask Jim to go to the Radiation Protection office and get another RPT to help.

- (Inference) Bob made judgment errors as he responded to this situation. Select ALL of Bob’s judgment errors below.

  - Deciding to review the last survey

  - Deciding to use his dose rate meter to determine whether the water was contaminated (correct)

  - Deciding to obtain a dose rate meter for the radiation survey, instead of a count rate meter

  - Deciding the water was not contaminated (correct)

  - Deciding to look for the source of the water

  - Deciding to operate the leaking valve (correct)

- (Inference) If the background radiation in the area was 310 CPM on a portable G-M count rate meter with a pancake probe, how should a contamination survey be conducted?

  - Using the portable G-M count rate meter with pancake probe to frisk the floor

  - Using the portable radiation survey meter with ionization probe to frisk the floor

  - Swipe the floor and use the portable G-M count rate meter to check the swipes (correct)

  - Swipe the floor and use the portable radiation survey meter to check the swipes

- (Inference) Which of the following methods would be the most effective in preventing additional personnel from becoming contaminated from the water in the hallway?

  - Make a plant-wide announcement

  - Establish protective clothing entry requirements for the Aux Bldg

  - Post boundaries around the contaminated area (correct)

  - Lock all doors to the area

- (Inference) Which explanation below explains why your above choice is the most effective method?

  - It provides a physical barrier to all unauthorized personnel

  - It provides local restrictions for the smallest area (correct)

  - It provides immediate awareness of all plant personnel

  - It provides for free movement throughout the Aux Bldg

- (Inference) Which of the following methods would be most effective in preventing the contamination from spreading?

  - Squeegee the water to one location and collect it with a wet vac

  - Use absorbent cloths to collect the water where it is (correct)

  - Wipe up the water with a dry cotton mop

  - Apply a strippable coating to the water and then remove

ACAD References

Note: ACADs listed in the RPT 243 instructors’ guide introduction document may also apply

ACAD

- 3.3.9 Contamination Control

- Describe techniques for controlling the spread of contamination to personnel and equipment, including the following: - Use of protective clothing - Packaging of contaminated materials - Use of containment devices - Control of leaks from radioactive systems - Decontamination

- Describe the devices used for containment of contamination during radiological work, such as drapes, glove bags, tents, drain bottles, berms, absorbents to contain liquid and catch containments

- 3.3.11 CONDUCT AND MONITORING OF RADIOLOGICAL WORK

- Describe precautions to be used when practical to control spread of radioactive contamination during radiological work, such as: - Use of containment devices - Special protective clothing requirements - Use of disposable coverings during job-site preparation

- 3.3.12 RADIOLOGICAL INCIDENT EVALUATION AND CONTROL

- Describe how to locate and track a radioactive plume.

- 3.3.13 DECONTAMINATION

- Describe the procedure to be followed when an individual is contaminated, including: - Documenting final results of decontamination

- Describe methods used for decontamination of areas within the plant, such as: - Mopping - Oil-impregnated wipes (masslin) - Wiping with damp rags - Vacuuming - Scrubbing with brushes

- Explain why area decontamination should begin at areas of lowest contamination levels and progress toward areas of high levels.

- Identify special precautions associated with disposal of materials used in decontamination, such as: - Wet mop heads - Liquids containing detergents - Vacuum cleaner dust - Mixed waste