ACADs (08-006)  Covered

| 3.2.3.8.4 | 3.2.3.23.5 | 3.2.3.23.6 | 3.2.3.23.8 | 3.3.15.1 | 3.3.15.2 |
| --- | --- | --- | --- | --- | --- |

Keywords

Air exhaust sampling, effluent air flow, sample check.

Description

This document provides references to readings and homework, as well as additional manuals to use for support, then provides scenario questions for students.

Supporting Material

Office of Environmental, Safety and Health: Radiological Control Technician Training Site Academic Training Study Guide Phase I, Project Number TRNG-0003.

Gollnick, D. (2006). Basic Radiation Protection Technology, 5th Ed. Pacific Radiation Corporation, Altadena, CA.

Spectrum Spectroscopy Techniques Lab Manual (Instructors and Student Versions).

Supplemental Lab Manual (instructors and Student Versions).

Environmental Sample Counting Using Proportional Counters

# Module 6: Perform Environmental Sample Counting Using Proportional Counters

Table of Contents:

Resources Key	48

Module Readings and Homework	48

Primary Scenario “Determine Gross Alpha/Beta Activity in Air Exhaust”	48

Transfer Scenario “Determine Gross Alpha/Beta Activity in Environmental Samples”	49

Module Assessment Items	50

Primary Scenario “Determine Gross Alpha/Beta Activity in Air Exhaust”	50

Suggested Labs	51

ACAD References	52

# Resources Key

| This refers to: | This reference: |
| --- | --- |
| ACAD | National Academy for Nuclear Training, Uniform Curriculum Guide for Nuclear Power Plant Technician, Maintenance, and Nonlicensed Operations Personnel Associate Degree Programs, ACAD 08-006. |
| DOE-SG | Office of Environmental, Safety and Health: Radiological Control Technician Training Site Academic Training Study Guide Phase I, Project Number TRNG-0003 Available at: http://nsedu.rnet.missouri.edu/docshare/. File is located under the Docs/General Curriculum/DOE materials folder. |
| G. | Gollnick, D. (2006). Basic Radiation Protection Technology, 5th Ed. Pacific Radiation Corporation, Altadena, CA. |
| Spectrum | Spectrum Spectroscopy Techniques Lab Manual (Instructors and Student Versions) |
| Supl. Lab | Supplemental Lab Manual (instructors and Student Versions) |

# Module Readings and Homework

Primary Scenario “Determine Gross Alpha/Beta Activity in Air Exhaust”

| Core Concept: Alpha/Beta Gross Counting Using Proportional Counter |  |  |
| --- | --- | --- |
|  | Homework (end of chapter) |  |
| Readings | Calculation Items | Non-calculation Items |
| G., Chap. 7, 254-260 DOE SG 1.13-14 through 1.13-17 DOE SG 2.19-7 though 2.19-8 | N/A | DOE SG 2.19.01 |
| Core Concept: Alpha/Beta Particle Discrimination Using Proportional Counter |  |  |
|  | Homework (end of chapter) |  |
| Readings | Calculation Items | Non-calculation Items |
| G., Chap. 7, 254-260 DOE SG 1.13-14 through 1.13-17 DOE SG 2.19-7 though 2.19-8 | N/A | G., Chap. 7, #12 DOE SG 2.19.01 |
| Core Concept: Air Particulate Monitoring |  |  |
|  | Homework (end of chapter) |  |
| Readings | Calculation Items | Non-calculation Items |
| G., Chap. 10, 416-427 DOE SG 2.06-6 through 2.06-20 | DOE SG 2.06.09 | DOE SG 2.06.03 through 2.06.08 |

| Core Concept: Calculation of Air Concentrations |  |  |
| --- | --- | --- |
|  | Homework (end of chapter) |  |
| Readings | Calculation Items | Non-calculation Items |
| G., Chap. 10, 423 DOE SG 2.06-6 through 2.06-20 | DOE SG 2.06.09 | N/A |

Transfer Scenario “Determine Gross Alpha/Beta Activity in Environmental Samples”

Refer to readings and homework for primary scenario above.

# Module Assessment Items

Note: If instructors wish to increase the difficulty of any item, then we suggest you use it as the basis for an in-class discussion, and / or require students to write an explanation for why a particular choice is correct.

Primary Scenario “Determine Gross Alpha/Beta Activity in Air Exhaust”

Assessment Items

You are a Radiation Protection Technician at the Nuclear Maintenance Facility (NMF). This facility specializes in the maintenance of contaminated nuclear power plant equipment. The work spaces at the NMF are ventilated with fresh air which is discharged to the environment after passing through HEPA particulate filters. The effluent air flow is continuously sampled with an isokinetic sample probe providing a representative volume of air drawn through an effluent sample filter to collect particulate material. Every week the radioactivity collected on the sample filter is collected and analyzed approximately 72 hours later to determine the amount of radioactive particulate NMF discharges to the environment. One of your routine job tasks is to collect the effluent sample filters for analysis.

1. (Inference) While you are performing the sampling evolution, you accidentally drop the filter paper when removing it from the sample holder. It lands on the floor with the sample side down. You carefully pick it up and place it in the protective envelope for analysis. What is the most important consequence of this occurrence?

A.) The radioactive contamination on the floor is most important due to the radioactive particles on the filter being a significant source of contamination.

B.) The larger amount of radioactive particulate discharged to the environment is most important due to some of the radioactive material on the sample filter being lost to the floor.

C.) The smaller amount of radioactive particulate discharged to the environment is most important due to some of the radioactive material on the sample filter being lost to the floor.

D.) The accuracy of the amount of radioactive particulate discharged to the environment is most important due to the sample results being inaccurate.(Correct)

2. (Prediction) In a gas-filled detector operating in the Proportional Region, how would a change in the detector voltage affect the number of electrons produced in the detector?

A.) As voltage decreases, the number of electrons increase

B.) As voltage increases, the number of electrons increase (Correct)

C.) As voltage decreases, there is little or no change in the number of electrons.

D.) As voltage increases, there is little or no change in the number of electrons.

3. (Explanation) Which of the answers below best explains how alpha and beta radiation each impact the circuitry of a gas-filled proportional detector?

A.) Alpha radiation produces smaller pulses of current in the detector circuitry than beta radiation because if its lower penetrating ability.

B.) Beta radiation produces larger pulses of current in the detector circuitry than alpha radiation because of its higher penetrating ability.

C.) Alpha radiation produces larger pulses of current in the detector circuitry than beta radiation because of its higher ionization potential. (Correct)

D.) Beta radiation produces larger pulses of current in the detector circuitry than alpha radiation because of its lower ionization potential.

4.) (Inference) Given the conditions in the scenario described above, which type of detector system provides the most accurate measure of effluent air radioactivity?

A.) A gas-filled G-M detector system is best because it is portable and permits quick and immediate results.

B.) A gas-flow proportional detector system is best because it measures current instead of pulses.

C.) A gas-filled G-M detector system is best because it responds indiscriminately to most types of radiation.

D.) A gas-flow proportional detector system is best because it discriminates between different isotopes. (Correct)

5.) (Explanation) Explain why this sampling method (Isokinetic with 72-hour decay)does not measure the actual amount of radioactive material discharged from the facility in the effluent air flow?

A.) Isokinetic sampling is not representative of the effluent air flow.

B.) The upstream HEPA filters remove the radioactive isotopes before they are discharged.

C.) The sample only contains long-lived isotopes when it is analyzed (Correct)

D.) The sample analysis is masked by short-lived isotopes before they decay away.

- (Prediction)  How would the number of calculated DACs in the effluent air change if an error in sample counting date resulted in the sample being counted one day early?

A.) The number of DACs would not change because only long-lived isotopes are being counted.

B.) The number of DACs would increase because there would be more activity due to short-lived isotopes.(Correct)

C.) The number of DACs would decrease because there would be less activity due to short-lived isotopes.

D.) The number of DACs would not change because only short-lived isotopes are being counted.

# Suggested Labs

Supl. Lab #16: Alpha-Beta Discrimination in Proportional Counters, page 13

Supl. Lab #17: Measurement of alpha energy spectrum, page 20.

ACAD References

ACAD

- 3.2.3 Radiological Survey and Analysis Instruments

- Identify the instruments available for performing contamination surveys such as the following - Proportional counters

- Explain the operating characteristics and use of the following radiological survey and analysis instruments: - Gross alpha counter - Gross alpha/beta counter

- 3.3.15 Environmental Monitoring

- Explain the purpose of the plant environmental monitoring program.

- Describe the methods used to conduct environmental monitoring, such as: - Air sampling - Vegetation sampling - Aquatic life sampling - Water sampling - Soil sampling - Milk sampling