Equipment Technician

| Task Sheet 4.3 | Network Wiring, Modules, Identify and Replaced Terminating Modules |
| --- | --- |
| Objective | Learn how to repair Network wiring, identify terminating modules |
| Tools and Equipment Needed | DVOM (Digital Volt Ohm Meter) Service Information Soldering Equipment Wire Stripping Tool |

NAME __________________________________________________________________

Year/Make/Model of equipment Being Worked on ________________________________

- In this exercise, you will learn how to repair network wiring.  You will also learn how to replace and program a module with a terminating resister inside of it.   If the wiring you are working on has an open or a short, it will be necessary to perform repairs to bring it back into service.

- Locate the wiring that is open or shorted.  You will need to repair by soldering the wiring and using shrink wrap to ensure that the wiring is well insulated.  Due to the fact that the wiring is twisted, it will be necessary to attempt to twist the wiring the same as it was before to minimize interference in the network.  Most CAN BUS wiring will have shielding over the wiring to minimize RFI (Radio Frequency Interference).  Using an appropriate soldering iron, perform repair to the wiring (do not forget to put shrink wrap over the wire before connecting and twist the wires before soldering.  Does the wiring have shielding over it?  _____________________________________________

- Have your instructor inspect and sign off to this point __________________________

- If the CAN network is shielded,  it will be necessary to repair the shielding and make sure the shielding is grounded properly.  If the original shielding is intact, try and preserve however, if it is not, it will be necessary to replace the shield or even replace the harness.  If the shielding is intact, repair and wrap with electrical tape.  Have your instructor sign off.  ______________________________________________________

- Which modules according to the description and operation of the service manual are the locations of the terminating resisters?  ___________________________________

- Using a DVOM, test the resistance of the CAN High and CAN low circuits.  What is the resistance reading? _____________________________________________________

- In the service information.  Is there any programming necessary when replacing either of these modules?  _____________________________________________________

- Why is it necessary to program modules to the equipment? _____________________________________________________________________

Turn this into your instructor