Equipment Technician

| Task Sheet 4.2 | Network Tests, Diagnostics, Scope Tests |
| --- | --- |
| Objective | In this task sheet you will demonstrate performing network tests, resistance checks and using a scope to test a Network. |
| Tools and Equipment Needed | Service Information Digital Volt/Ohm Meter 2 Channel Oscilloscope |

NAME __________________________________________________________________

Year/Make/Model of equipment Being Worked on ________________________________

- Using a suitable scan tool, perform a network test.  List all the modules that responded to the network test. _______________________________________________________

- Are there any modules that did not respond? _______________________________

- Are there any modules that are optional for this vehicle? _____________________________________________________________________

- How would you tell if a vehicle is equipped with a module that is optional but not responding? __________________________________________________________

- Perform a self-test. Are there any codes in any of the modules? _________ List any codes ____________________________________________________________________

- Are there any network codes? __________________________________________________

- How do you know if a code is a network code? ____________________________________________________________________

- Set your multimeter to Ohms. Using your multimeter, check resistance across the 2 pins of the Data Link Connector.

- Which 2 pins are the CAN BUS network in the Data Link Connector? _____________

- What is the measured resistance that you got? ______________________________

- Is this a good resistance? ________________________________________________

- What would happen if there were an open in 1 of the terminating resisters? _____________________________________________________________________

- Using a 2 channel Oscilloscope, set each channel to 500ui division and a speed of 200k/s. On each channel, connect the positive lead to one of the pins on the DLC (Data Link Connector) and the other to chassis ground. Turn key on and wait 1 minute.

- What is the bias voltage (base voltage) on each circuit? _______________________

- Start the vehicle. What is the high voltage on each circuit? ____________________

- What do you notice about the 2 patterns? Do they look the same? Are they opposites of each other? _______________________________________________

- What do we call the groupings of messages coming through? ____________________________________________________________________

- Try changing the speed and time divisions of the screen.  Try freezing the screen to see the pattern close up.  Show your instructor ____________________________________

Turn into your instructor