Zoning Lab Form

7 questions at 4 points a piece – 28 total points

Name:      
Date:

- Paste the screen shot taken in the “Capture and view data transmitted in the Cell-Area zone“ part of the lab into this question:

- What does the value in Register 0 probably represent?

(-4) The value in register 0 represents the current tank level. You can determine this by looking at the HMI and noting that the values in the Pump on at and pump off at fields match what is shown in registers 1 and 2. The only other value in the system that is something other then true/false is the current tank level.

- What does the value in Register 1 probably represent?

(-4) The value in register 2 indicates when the pump should be turned off. This matches the pump off at field from the HMI.

- Paste the screen shot taken in the “Capture and view data transmitted in the Manufacturing zone“ part of this lab into this question:

Sound have captured ICMP and broadcast traffic only.

- Why was the network ping traffic between the Kali-Modus system and the PLC captured but the data between the PLC and other ICS systems was not?

(-4) Devices can only sniff data from networks they are attached to or that is addressed to or from their own MAC address. The Kali-Modus system was moved to a different network then the PLC system. This allowed it to view ping traffic from itself to the PLC but prevented it from viewing traffic between the PLC and OPC server.

- If using proper zoning techniques is more secure why might companies not configure their systems using this technique?

(-4) The main reason companies might not choose to secure their networks using zoning is because it is more complicated and expensive then just putting all of the systems on the same network.

- If you were to capture data on the Cell-Area zone and you consistently observed the following data what might you conclude regarding the functionality of the cooling system?

(-4) This screen shot probably means that the pump has failed or has been turned off, and the cooling tank is empty. The settings show that the pump should turn on when the cooling level gets down to 10% of the tank and should turn on when it gets up to 60% of the tank. The current level of the tank is 0% showing that it is empty.