# BUILDING A WEB SERVER FOR I/O CONTROL

# Electronic Communication

### Acknowledgements

Subject Matter Expert:  Jesus Casas, Professor, Austin Community College, Texas

### Purpose

The purpose of this lab is give an instructor the necessary information and instructions for setting up a web server through which server I/O can be controlled.

### Overview

Web-based instrumentation and control has seen plenty of growth in the last decade.  While dedicated and proprietary instrumentation and control systems are perfectly fine, web-based systems have several advantages that make them ideal in certain applications.  Web-based systems allow remote access and control that can be performed from any computer that is connected to the Internet.  Since the web server “serves” the web pages, there is no need for special software at the client computer.  Web-based systems can also be configured to send status reports, email, and save data as a file for review at a later date.

Even though web-based systems have advantages that are very appealing, a major disadvantage is that any system connected to the Internet is prone to “hacker” attacks.  Therefore security measures must be incorporated to remain secure.

This project started from the need to control home lighting via the Internet.  There are companies that provide modules for such applications but the cost is invariable high; and in most cases, the modules are very limited in resources such as the amount of program storage available, speed, and functionality.  However, the advantage to these modules is that they are small in size, and fairly simple to program and configure.  Some of these modules are as small as one square inch.

Even though web-based I/O control modules are available, the challenge of building a more robust system from a PC has a big appeal.  Not only can the PC do all that the smaller web-based modules can do, but due to the added power and resources, it can run intricate programs and stream audio/video if needed.

This document provides the instruction necessary to set up, configure, and load the necessary scripts that will allow control of the server’s parallel port (378h) over IP address.  The web server may be connected to the Internet, provided some security measures are implemented.  The instruction provided herein provides a starting-point for more challenging projects.

### Systems Rationale

It is important for the students to have an understanding of web server systems.  A web server system is comprised of both hardware and software that go hand in hand to serve a web-page and respond to user inputs.  The student should be exposed to all the underlying software and hardware details, and how they work together to make a web server system work.

### System Concepts

This system covers the following system concepts (signified by an X):

_X_ 	S1. A system can be defined in terms of its functional blocks i.e., a “structured functional unit.”

_X_ 	S2. A system has a purpose, transforms inputs into outputs to achieve a goal.

_X_ 	S3. A system is defined by the flow of materials, energy and information, between its functional units.

__ _	S4. A system may be open or closed. In an open system additional inputs are accepted from the environment.

___ 	S5. A system is more than the sum of its parts. Individual components can never constitute a system.

_X_ 	S6. A system provides feedback to the operator and services to the user. Some system functions may involve operator action.

___ 	S7. Systems have unique problems.

### Learning Outcomes

For a full course SLOs, click the link and click SLO tab.

http://www.esyst.org/Courses/DC-AC/_delivery/index.php }

- Describe how the Internet works.

- Give the basic specifications for an Ethernet LAN, state the standards used and describe the data formats and transmission media commonly used.

- Explain the TCP/IP protocols.

### Prerequisite Knowledge & Skills

The user of this documentation should have some programming skills; preferably in HTML code and Perl.  Also, a basic understanding of microprocessors and electronics would be helpful.

There are numerous web-sites that provide tutorials and examples of HTML code and Perl scripts.  Having a reference book on either or both is also a plus.

### Learning Objectives

Relevant knowledge (K) or skill (S) or attitudes (A) objectives include:

K1.  The student should be able to identify the hardware components (server, router, client) involved in establishing communications over IP address.

K2.  The student should be able to identify and describe the software components (web server software, translator, software driver, scripts, HTML) required in performing data I/O over IP address.

### Instructional Setting

This lab has been designed to take place in e anywhere where there are a few PCs that are or can be networked.

Instructor Setup/Cleanup:

This will vary depending on the particular lab room arrangement and storage systems available at your institution.

Student Lab Performance:

It should take students approximately 6 hours to work through the entire lab.

Student Lab Deliverables:

None

### Materials, Equipment, & Supplies Needed

| Item | Quantity |
| --- | --- |
| Computer running Windows XP and Internet Explorer | 1 |
| LEDs | 8 |
| 220 Ohm Resistors | 8 |
| Abyss Web Server software (see Procedure) | 1 |
| ActivePerl software (see Procedure) | 1 |
| Inpout32.dll driver (see Procedure) | 1 |
| Project software (see Procedure) | 1 |

### Special Safety Requirements

None.

### Lab Preparation

- Decide upon assessment and grading criteria.  Insert this information at the end of student lab guide.  Sample criteria have been included in the grading and assessment area, which can be found at the end of this instructor guide.

### Quick Tips

None.

### Introduction

A web server sends out HTML code that a web browser interprets and displays as a “web page”.  A web page by itself is static and really only displays content.  Any transfer of data between the client and server requires the use of a Common Gateway Interface (CGI) script.  A CGI script is an external program that the server runs to read the information that the browser has sent and performs some function with that data.  Upon completion of its task, the script is exited.

Perl is the most popular language for CGI scripting.  It provides dynamic web page generation and is unrivalled in its text-handling capabilities, its ease of scripting, and is relatively fast.

ActivePerl is used to handle the CGI scripts in this project.  It contains modules that allow added functionality.  One of the modules that will be used is Win32 API.  This module allows us to import and call arbitrary functions from Win32 Dynamic Link Libraries (DLLs).  Another module that will be used is the CGI module.  This module allows the running of CGI scripts that collect Form data from the browser.  Two great resources for CGI scripts and module information can be found at www.cgi101.com and www.cpan.org .

The main functionality of this project is to provide web-based control of the server’s Parallel port, more specifically the Data register of the Parallel port, operating at an address of 378h.  The base address of operation for the Parallel port can be selected through the system BIOS.  Please note, that by default, most Parallel ports are factory configured for operation at 378h.  The software included for this project has been written exclusively for operation at 378h.

At the physical output of the Parallel port, solid state relays can be attached to control 110VAC devices such as household lights, appliances, and environmental controls.  Initially LEDs may be connected to get a visual indication that the system is operating correctly.

Access to the hardware (Parallel port) will be through a DLL driver.  Windows XP does not allow application software to directly write to system hardware; therefore “calls” will be made to a DLL driver via a CGI script ran with ActivePerl.  The DLL driver Inpout32 will be the interface between the application software and the hardware.

Projects

Project 1 – This project provides control of D0 at 378h and the ability to play a music clip.

Project 2 – This project is the same as Project 1 but includes the functionality to display server date and time.

Project 3 – This project provides control of all eight bits (D7 - D0) of the Data register using an individual web page for each bit.

Project 4 – This project provides control of all eight bits (D7 – D0) of the Data register but uses a single web-page for control of all eight bits.

Project 5 – This project provides control of all eight bits (D7 – D0) of the Data register with verification.  A status check feature is also provided.  It also includes the functionality to display server date and time, and play a music clip.

Project 1 Files

Project 1 includes the following files:

| File Name | Description |
| --- | --- |
| Index.htm | This is the first file that the web server serves. All other files are linked to this file in one way or another. The web-page that is Index.htm can be seen with any web browser, and modified with Internet Explorer or any HTML writing tool. |
| Home_automation.gif | This is a picture file. |
| For-Once-In-My-Life.mid | This is a midi sound file. |
| On.pl | This is a Perl script that turns on D0 of the Parallel port. This file can be viewed and modified with a simple text editor such as Windows Notepad. |
| Off.pl | This is a Perl script that turns off D0 of the Parallel port. This file can be viewed and modified with a simple text editor such as Windows Notepad. |

The Index.htm file appears as follows when seen through a web browser.

The source code for Index.htm is:

<html>

<head>

<title>Untitled Document</title>

<meta http-equiv="Content-Type" content="text/html; charset=iso-8859-1">

</head>

<body bgcolor="#FFFFFF" text="#000000">

<div align="center">

<img src="home_automation.gif" width="357" height="267"><br><br>

<b><font color="#000000" size="6">Welcome to Home Control!</font></b><br>

<font color="#0000ff" size="4"><i>powered by Abyss Web Server</i></font>

<hr>

</div>

<div align="left">

<b><font size="4">LED Control</font></b><br>

<form action="on.pl" method="post">

<input type="submit" value="ON">

</form>

<form action="off.pl" method="post">

<input type="submit" value="OFF">

</form>

</div>

<hr>

<div align="center">

<font size="5">Click <a href="For-Once-In-My-Life.mid">Here</a> for a short music clip </font> </div>

</body>

</html>

Analyzing the web-page and HTML code, one can see where the picture and music files are placed and how they are called.  A web-page can display different formats of picture files such as jpeg, bmp, gif, and others.  Even though a midi file is used for this project, an MP3 file or even a video file can be used as well.

One can also see that when the “ON” button is clicked, the file “on.pl” is executed.  The “on.pl” file is a Perl script which uses the Win32 API module to read and write values to the Parallel Port.  The following is the code for “on.pl”.

#!c:/perl/bin/perl.exe

use strict;

use warnings;

use Win32::API; #load API module to interface DLL's

my $GetPortVal = Win32::API->new('inpout32', 'Inp32', ['I'], 'I') or die

"Couldn't create Win32::API object: $! $^E";

my $SetPortVal=  Win32::API->new('inpout32', 'Out32', ['I','I'], 'I') or die

"Couldn't create Win32::API object: $! $^E";

my $output=$SetPortVal->Call(0x378,0x01);

my $input= $GetPortVal->Call(0x378) & 255;

print "Content-type:text/html\n\n";

print "<html>\n";

print "<head><title></title></head>\n";

print "<body>\n";

print "<p>Value sent = $output</p>\n";

print "<p>Value read = $input</p>\n";

print "<p>If both results are 1, then the LED is On!</p>\n";

print "</body>\n";

print "</html>\n";

The “off.pl” file is exactly as above except for the value that is sent to 378h in the following line:

my $output=$SetPortVal->Call(0x378,0x01);

It has been changed to send out a value of 0:

my $output=$SetPortVal->Call(0x378,0x00);

The Parallel Port

The Data register is comprised of Pins 2 thru 9 (D0 – D7 respectively).  Pins 18 thru 25 are Grounds.  Connect the LEDs and resistors to the Parallel port in the following manner.  The LEDs will be used as visual indicators.

Procedure:

1.)	Read and follow the directions in the document, “About and Installing INPOUT32”.

2)	Read and follow the directions in the document, “Installation of ActivePerl”.

3).	Read and follow the directions in the document, “Installing and Configuring Abyss Web Server”.

4)	Download the files of Project 1 to the folder “htdocs” found under “C:\Program Files\Abyss Web Server”.

5)	Connect the web server computer to a router and start Abyss Web Server.

6)	Obtain the web server’s IP address.

7)	On a different computer that is still part of the network that the server is connected to, access the web server’s web page.  Turn “On” and “Off” bit D0 of the Parallel port and verify the state using the LEDs.

8)	Once Project 1 is understood, proceed to the other projects.

9)	At some point, restricted access to the I/O control web-page should be implemented.  Read and follow the directions in the document, “Controlling Web Access”.

Final Thought

All project information and documentation is provided in the hope that it will lead to more creative and challenging web-based projects.