# iCREAT I: Module 6 - LAB 1- Arduino Servo Motors

# Objectives

- Learn how to control the position of a servo motor with your Arduino and a potentiometer.

- Learn how to change direction (sweep) back and forth using a servo

- Learn how to generate random numbers

# Background / Scenario

Servo motors have been around for a long time and are utilized in many applications. They can be used to operate remote-controlled or radio-controlled toy cars, robots and airplanes. Servo motors are also used in industrial applications, robotics, in-line manufacturing, pharmaceutics and food services.

A Servo is a small device that incorporates a two wire DC motor, a gear train, a potentiometer, an integrated circuit, and an output shaft. Of the three wires that stick out from the motor casing, one is for power, one is for ground, and one is a control input line

The Servo Library allows Arduino boards to control a variety of servo motors, more information can be found at the Reference for the Servo Library page.  This library allows an Arduino board to control RC (hobby) servo motors. Standard servos allow the shaft to be positioned at various angles, usually between 0 and 180 degrees.

In this lab you will learn and experiment with servo motors and the Arduino microcontroller.

# Required Resources:

- Arduino UNO board connected to a computer

- Appropriate USB cable

- Access to the Arduino IDE

- Breadboard

- Servo Motor

- 2 LEDs (RED, GREEN)

- 10KΩ potentiometer

- Resistors (220Ω, or 330Ω, or 100Ω)

- Jumper wires

- Access to the iCREAT course

Part 1. “Knob” Tutorial (OPTIONAL)

We will control the position of an RC (hobby) servo motor with your Arduino and a potentiometer. In this section you will follow the instructions for the Arduino “Knob” Tutorial.  You could use 3 or more servos and potentiometers to build your own robotic arm using cardboard pieces, or design and 3D print the necessary parts..

- Follow the diagram to create the circuit:

- Download the code by clicking on the [Get Code] link, and read and understand the code. Feel free to experiment!

- Have fun and experiment by adding an LED (with a resistor of course!) to your circuit so that when the value of the potentiometer changes and the servo moves, the intensity of the LED also changes.

Show your completed diagrams and working programs before you move to the next section.

Part 2. “Sweep” Tutorial

We will Sweep the shaft of a RC servo motor back and forth across 180 degrees. In this section you will follow the instructions for the Arduino “Sweep” Tutorial.

- Follow the diagram to create the circuit:

- Download the code by clicking on the [Get Code] link, and read and understand the code.

- Have fun and experiment by adding an LED to your circuit,  add another servo that moves to the opposite direction, and use the map() function.

- Show us your completed diagrams and working programs before you move to the next section.

Part 3. Random Servo

For this exercise you will make a servo to randomly turn left and right. You will use the functions randomSeed(x) and random(x,y) to accomplish this task.

- Follow this diagram to create the circuit:

- Follow the code provided and modify it to print the output generated on the serial monitor. Run and test it.

#include <Servo.h>
Servo servoM;
long randomNumber;

void setup() {
	servoM.attach(10);
	randomSeed( analogRead(A0) );
}
void loop() {
	randomNumber = random(0,181);
	servoM.write(randomNumber);
	delay(500);
}

- Change your circuit to add two LEDs to show if the random number generated is odd (RED) or even (GREEN).

- Show us your completed diagrams and programs.

Tutorials, Examples and help at https://www.arduino.cc/en/Tutorial/BuiltInExamples