# iCREAT I: Module 6 - LAB 2- Arduino Motor Shield

# Objectives

- Learn what a library is and how to install them on the Arduino environment.

- Learn how to perform a manual installation of an Arduino library.

- Learn how to install an Arduino motor shield

- Get familiar with Adafruit Motor Shield V1.0

- Get familiar with the full documentation for the “Adafruit Motor Shield”

- Get familiar with the Adafruit motor shield to control a servo, and 2 DC motors

# Background / Scenario

Once you are comfortable with the Arduino software and using the built-in functions, you may want to extend the ability of your Arduino with additional libraries. Libraries are a collection of code that makes it easy for you to connect to a motors, servos, sensor, display, module, etc. The built-in Arduino libraries and some of these additional libraries are listed in the reference. To use the additional libraries, you will need to install them. There are three ways for installing an Arduino library: 1) Using the Library Manager, 2) Importing a .zip Library, or 3) Manual installation.

A Motor Shield is a driver module that allows you to easily use a microcontroller to control the working speed and direction of motors. The Adafruit Motor Shield is a great and quick way to control DC motors, servos or even stepper motors. It has the capability of controlling up to 2 stepper motors, 4 DC motors, and 2 servos. In this lab you will learn and experiment with the Adafruit Arduino Motor Shield  and the Arduino microcontroller.

# Required Resources:

- 1x Arduino UNO board connected to a computer

- 1x Appropriate USB cable

- 1x Breadboard

- 1x Adafruit Arduino Motor Shield V1.0

- 1x car chassis with 2x DC Motors

- 1x Servo Motor

- 1x External power supply (4x1.5v battery pack, or a 1x9v battery pack)

- Jumper wires

- Access to the Arduino IDE

- Adafruit Motor Shield Library

Part 1. Installing the Adafruit Motor Shield Library

(This MUST be done on the PC before part 2 )

In this section you will learn how to manually install a third-party library on your Arduino IDE environment. Full documentation for the Adafruit Motor Shield V.1.0 could be found here: “Adafruit Motor Shield”. To make life easier for the final project, you may use the book titled: “Make an Arduino-Controlled Robot” by Michael Margolis as a reference.

- To install the library, first quit the Arduino IDE.

- Click on the following URL to have access to all the material and sample code used in the book:

http://shop.oreilly.com/product/0636920028024.do

- Use this direct link to download the library. On the upper right-hand side click on the “Download ” icon next to “Find file” icone, which will  download a ZIP file.

- On Windows, the ZIP file will be downloaded to the “Downloads” folder:

“C:\Users\YOUR-USER-NAME\Downloads”

- The downloaded file name should look like “0636920028024-master.” Extract the ZIP file.

- For simplicity’s sake, rename the extracted folder to “master”.

- Copy all the libraries contained in the  “master\libraries“ folder into your own libraries folder. Under Windows, it will likely be called "My Documents\Arduino\libraries". For Mac users, it will likely be called "Documents/Arduino/libraries". On Linux, it will be the "~/Arduino/libraries" folder.

- Copy the sample files contained in the  “master“ folder (except “master/libraries”) into your own Arduino folder. Under Windows, it will likely be called "My Documents\Arduino". For Mac users, it will likely be called "Documents/Arduino". On Linux, it will be in “~/Arduino”.

- Also, in Module-6, “MotorTest-iCREAT” and “MotorTest-2wd-iCREAT” are a modified version of the originals “MotorTest” and “MotorTest-2wd”. Add both folders to; on Windows "My Documents\Arduino", on MAC "Documents/Arduino" and on Linux it will be under “~/Arduino”.

- Restart the Arduino IDE. Make sure the new library appears in the “Sketch->Include Library” menu item of the software. That's it! You've installed a library!

- Follow the diagram to create the circuit (do not forget the Arduino):

Part 2. Testing the Adafruit Motor Shield Library

- In order to complete part-2 you must have the libraries in part-1 on your current machine.

- Click on File->Sketchbook->MoterTest2WD

In order to do your testing use the car chassis provided. Read and understand the code, and make sure you perform the proper adjustment (motors, servos, output lines, etc).

2.  Have fun and experiment with your code.

3.  Show your completed diagrams and working programs before you move to the next section.

- Tutorials, Examples and help at https://www.arduino.cc/en/Tutorial/BuiltInExamples

- This tutorial is partially based on the book titled “Make an Arduino-Controlled Robot” by Michael Margolis  (O’Reilly). Copyright 2013 Michael Margolis, ISBN (978-1-4493-4437-5)